Steven Martin

Analog Devices, Inc.

Labcomm

V1

A Peripherals Protocol

July 6, 2023

Labcomm

INTRODUCTION

Labcomm is a simple, lightweight, and fast binary protocol for communicating with electronic hardware in the lab such as customer demonstration boards. It is very similar to an ethernet frame packet. One major difference is that electrical integrity features such as CRC checking and clock training have been removed since the intended transport mechanisms of USB or high level TCP/IP should have these issues covered already.

Figure 1 shows a pictorial representation of a Labcomm packet.

┌──────────┬─────────┬────────────────┬───────────┬──────────────┬─────────────┬──────────┐

│ PREAMBLE │ VERSION │ DESTINATION ID │ SOURCE ID │ PAYLOAD SIZE │   PAYLOAD   │ ENDAMBLE │

└──────────┴─────────┴────────────────┴───────────┴──────────────┴─────────────┴──────────┘

│◄───10B──►│◄──1B───►│◄───2B────►│◄──────2B──────►│◄─────4B─────►│◄─SIZE(P/L)─►│◄───4B───►│

Figure 1. A Single Labcomm Packet

PREAMBLE

The 7-byte preamble is a symbol that transceivers on each end of the wire scan for asynchronously. While not intended to deal explicitly with electrical noise problems, by searching for the preamble asynchronously, it is possible for the transceivers to, at least, attempt to recover from packet start/stop faults. Although this is unlikely to occur in the case of a USB cable for various reasons, the possibility for added robustness exists, nonetheless

The ten-byte preamble exceeds the size of the meta-data between it and the payload (9 Bytes). Therefore, it should not be possible for preamble to inadvertently be detected with the meta-data. Likewise, asynchronous preamble detection is blanked when the parser (state) is receiving the payload.

For want of any other random numbers, the preamble is defined as the 80-bits of the ASCII string: “L@BC0MmADI”. To ease debugging during human interaction, this is easily recognizable in debug printouts and serial port sniffers.

The numeric values of the preamble are:

| ASCII | L | @ | B | C | 0 | M | m | A | D | I |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Decimal | 76 | 64 | 66 | 67 | 48 | 77 | 109 | 65 | 68 | 73 |
| Hex | 0x4C | 0x40 | 0x42 | 0x43 | 0x30 | 0x4D | 0x6D | 0x41 | 0x44 | 0x49 |

VERSION

The 1-byte version field falls under the category of meta data representing the Labcomm version. If it is necessary to augment this protocol in the future, the version field can be used to alert the host and or client (and possibly the user) that the host and client are incompatible.

DESTINATION ID

The destination ID is a 2-byte identifier representing the sendee’s address. Two bytes supports 65,536 possible endpoints or clients. Endpoints represent a breakout of individual resources to be addresses at either end of the wire (USB presumably). Examples of endpoints might be IC serial ports such as I2C, SPI etc. Other endpoints may be: ENABLE pins, RST or PGOOD pins, other digital I/O, other integrated circuits such as EEPROM chips, temperature sensors, muxes, etc.

SOURCE ID

The source ID is a 2-byte identifier representing the sender’s address. Two bytes support 65,536 possible endpoints or clients. Endpoints represent a breakout of individual resources to be addresses at either end of the wire (USB for example). Examples of endpoints might be IC serial ports such as I2C, SPI etc. Other endpoints may be: ENABLE pins, RST or PGOOD pins, other digital I/O, other integrated circuits such as EEPROM chips, temperature sensors, muxes, etc.

LABCOMM ID

The address 0 is reserved for Labcomm itself. If a message can’t be sent, for example, it’s declared size exceeds that supported by the parser on the end of the wire, Labcomm reserves the to send back an unsolicited message indicating the fault.

ID REGISTRY

Ideally there will be a registry of endpoint IDs declaring which product an endpoint is associated with and whether the endpoint has general appeal or is fully custom for one project.

PAYLOAD SIZE

The 4-byte payload size field informs the receiver of the expected payload size. Labcomm has both a payload size field and an endamble field making one of them redundant. Without the payload size it is possible to compute the payload size by subtracting the size of the preamble, source and destination sizes and the endamble size. Including the payload size field and checking for the endamble field ensures that no part of the next preamble could ever be interpreted as payload data. The payload is bracketed by the payload size field and the endamble.

PAYLOAD

The payload has variable size ranging from 0 to 4,294,967,296 bytes. The payload may vary from as simple as a single byte of 0x01/0x00 for ON/OFF to a rich menu of commands. Typically, the first byte of the payload will contain the local endpoint command to be executed and the structure of the endpoint must be documented and agreed upon between the communicating endpoints.

ENDAMBLE

The 4-byte endamble is an identifier token used to denote the end of a packet. As described in the payload size paragraph, the presence of the endamble combined with the payload size, gives a concrete place for the protocol parser to look for the end of a packet. If, following the payload size passed the payload, the endamble is not found, the message is rejected.

The endamble is the binary values of the ASCII string: “TeoM” meaning: The End of Message”.

The numeric values of the endamble are:

| ASCII | T | e | o | M |
| --- | --- | --- | --- | --- |
| Decimal | 84 | 101 | 111 | 77 |
| Hex | 0x54 | 0x65 | 0x6F | 0x4D |

USEAGE EXAMPLES – Stowe Demo Board BCB606

There needs to be a Labcomm parser on each end of the wire. PyICe supports Labcomm through its:

lab_interfaces.interface_labcomm_raw_serial()

and

twoWireInterfaces.i2c_labcomm() and i2c_labcomm_softport()

A PyICe parser for Labcomm and for Arduino can be found in a local PyICe checkout at:

\PyICe\deps\Labcomm\labcomm.py.

\PyICe\deps\Labcomm\arduino\labcomm.cpp.

\PyICe\deps\Labcomm\arduino\labcomm.h.

The Arduino files offer the additional service of a Mailbox{} class. The Mailbox class has adjustable inbox and outbox sizes as each endpoint will have different requirement. It also contains a data structure to represent various states of the mailbox as shown below along with methods get_packet() and send_packet():

uint16_t        my_id;

uint16_t        to_id;

uint16_t        from_id;

uint32_t        inbox_msg_size;

uint32_t        outbox_msg_size;

uint8_t         inbox_status;

uint8_t         outbox_status;

bool            inbox_overflow;

uint8_t         inbox[inbox_size];

uint8_t         outbox[outbox_size];

A complete example of the Labcomm protocol embedded in an Arduino M0 (processor ATSAMD21, 32bit, 48MHz) and connected to a PyICe GUI can be found in the Stowe Demo Board Project. Additional documentation can be found there in the file BCB606 Firmware Explained.docx. Likewise, the PyICe driver BCB606.py found in the folder /PyICe_driver and will be found to be like any other instrument driver in PyICe.

Aside from the requisite main file in the firmware stack BCB606.ino and BCB606.h, all projects will need a version of a “Post Office” (BCB606_postoffice.cpp/.h). The post office creates all instances of mailboxes in the firmware stack and performs the two services of posting outgoing mail and receiving incoming mail to/from the Labcomm parser.