PRACTICAL NO.2

- Familiarizing Quantum GIS: Installation of QGIS, datasets for both Vector and Raster data,

- Maps.

Ans)

OUTPUT:

Step1: Open QGIS-> Select Project→ New Project

Step2: Select Layer → Create Layer → New Shapefile Layer

Step 3: Set File name:kudal_area, Select Geometry type:polygoan, Set CRS: EPSG:4326 – WGS 84

New Field- add column name, set data type and length.

Step 4: To plot Polygon features -> Select the Polygon Feature from layer -> Click Toggle Editing

Button → Click on Add Polygon-> set attribute values.

Step 5: To set Style property -> Right click on layer -> click on control feature Symbology -> select

pattern or set color -> ok

Step 6: Right click on layer -> click on control feature labeling -> select Single Labels -> set

value=area_name ->ok

Step 7: Repeat same steps to add another polygon layers.

Step8: Select Layer → Create Layer → New Shapefile Layer

Step 9: Set File name:roads, Select Geometry type:line, Set CRS: EPSG:4326 – WGS 84

New Field- add column name, set data type and length.

Step 10: To plot line features -> Select the line Feature from layer -> Click Toggle Editing

Button → Click on Add line-> set attribute values.

Step 11: Right click on layer -> click on control feature labeling -> select Single Labels -> set

value=area_name ->ok

Step 12: To set Style property -> Right click on layer -> click on control feature Symbology -> select

pattern or set color -> ok

Step 13: Repeat same steps to add another line layers.

Step 14: Select Layer → Create Layer → New Shapefile Layer

Step 15: Set File name:schools, Select Geometry type:multipoint, Set CRS: EPSG:4326 – WGS 84

New Field- add column name, set data type and length.

Step 15: To plot multipoint features -> Select the multipoint Feature from layer -> Click Toggle Editing

Button → Click on Add multipoint -> set attribute values.

Step 16: To set Style property -> Right click on layer -> click on control feature Symbology -> select

pattern or set color -> ok

Step 17: Right click on layer -> click on control feature labeling -> select Single Labels -> set

value=area_name ->ok

Step 18: Repeat same steps to add another multipoint .

- Calculating line lengths and statistics

- Add Layer>Add Vector  Layer>Source> Addfile (IND_adm.zip )s

- Topology selection

- After adding (IND_adm.zip)

- Add Layer>Add Vector Layer->Source->Add file(IND_rrd.zip)

- Topology selection

- After adding (IND_rrd.zip)

- Opening attribute table

- Adding new field to attribute table using ‘ field calculator’

- Adding ‘road’ for calculating road length in kilometers(length / 1000)

- After adding road field

- Using vector analysis tools   vector->analysis tools ->basic statistics for fields

- Select the field and hit run

- View the logs

- Final result